Pond Catchment Analysis Backend
Technical Report

Name: G.Nikhilesh

ID:12340850

Course: CSD

Terrain Analysis, Hydrological D8 Routing, and Automated Pond Planning

| GitHub Repository | https://github.com/nikhilesh-git/pond-planning-system |
| --- | --- |
| Working API Endpoint (Remote) | http://10.1.75.51:3253/analyzeContour http://10.1.75.51:3253/docs |
| Local Development URL | http://127.0.0.1:3000/analyzeContour |
| API Framework & Protocol | FastAPI / Uvicorn (HTTP POST multipart/form-data) |

# 1. Working API Route & Deployment

The backend API service is deployed on the remote server and actively listening on port 3000:

Remote Server Base URL: http://10.1.75.51:3253

Analyze Contour Route: http://10.1.75.51:3253/analyzeContour

Interactive API Documentation (Swagger UI): http://10.1.75.51:3253/docs

# 2. Catchment Estimation Approach

The system performs a fully dynamic, multi-stage geospatial and hydrological pipeline to analyze terrain directly from raw contour data without relying on hardcoded coordinates.

• KML/KMZ Parsing: The uploaded contour file is parsed using high-speed 'lxml.etree'. LineStrings and their associated elevation values (Z attributes) are extracted with zero manual intervention.

• Coordinate Projection (UTM Transformation): Using 'pyproj', the algorithm automatically detects the optimal local Universal Transverse Mercator (UTM) zone from the bounding box of coordinates. Geographic coordinates (WGS84 Latitude/Longitude) are projected into metric Cartesian units (meters), ensuring high precision for distance, slope, and catchment area calculations.

• Digital Elevation Model (DEM) Generation: Contour point data is rasterized into a high-resolution 2-meter regular grid. SciPy's 'griddata' (Linear interpolation with nearest-neighbor extrapolation) constructs a continuous 2D surface elevation matrix z(x,y).

• Hydrological Routing (D8 Flow Algorithm): The D8 (Deterministic 8-neighbor) flow direction model determines the path of water descent across every cell. Flow accumulation is computed to simulate how surface runoff converges downstream.

• Identification of Natural Sinks (Depressions): Local minima where water naturally gathers without immediate outward flow paths are marked as candidate sinks. For each candidate sink, upstream tracing determines the full contributing catchment area.

• Multi-Stage Waterbody & River Exclusion Pipeline: To prevent selecting invalid locations (e.g., inside existing rivers, floodplains, or edge artifacts), candidate sinks pass through rigorous filtering:
  • Boundary Buffer Exclusion: Rejects sinks within 200m of the DEM edges.
  • Elevation Band Filter: Excludes sinks in the bottom 10% elevation range.
  • Flow Accumulation Filter: Sinks sitting on or directly adjacent to major river channels are rejected.
  • Flat Waterbody Rejection: Sinks within extensive flat zones (>1200 sqm) are flagged as existing lakes/reservoirs and excluded.

• Multi-Factor Suitability Scoring: Remaining candidate sinks are scored using a weighted suitability index:
  Total Score = (0.30 × Elev_Score) + (0.40 × Catchment_Score) + (0.30 × Accum_Score)
  - Elevation Score favors upland sites to enable gravity irrigation and avoid riverbeds.
  - Catchment Score rewards moderate, balanced catchments to prevent overflow risk.
  - Accumulation Score favors true micro-depressions over high-volume drainage lines.

# 3. Demonstration on Provided Contour Map

The API was validated by submitting the standard dataset (contours_1m.kml) to the /analyzeContour endpoint. The results obtained are as follows:

| Optimal Latitude | 21.25205° N |
| --- | --- |
| Optimal Longitude | 81.30924° E |
| Sink Elevation | 290.0 meters |
| Estimated Catchment Area | 772.0 – 780.0 m² |

Hydrological Evaluation of Selected Site:
The selected site sits at an elevation of 290.0m on an upland micro-depression. It successfully avoids the lower elevation river corridor (286m) that runs across the area. The catchment area provides sufficient inflow without risking structural embankment failure or heavy silt deposition.

# 4. Complete API Documentation

POST /analyzeContour

Uploads a KML or KMZ contour map and returns the computed optimal pond location along with its catchment area.

## Request Specifications

HTTP Method: POST

Content-Type: multipart/form-data

Body Parameters: file (Binary .kml or .kmz file, Required)

## Response Specification

Status Code: 200 OK

Content-Type: application/json

Example JSON Response Body:

{ "pond_location": { "latitude": 21.252048140122685, "longitude": 81.30923688499112, "elevation": 290.0 }, "catchment_area_sqm": 772.0 }

## Example cURL Command

curl -X POST "http://10.1.75.51:3253/analyzeContour" \ -H "accept: application/json" \ -H "Content-Type: multipart/form-data" \ -F "file=@contours_1m.kml"

Screenshots:

AI Usage:
Claude, Chatgpt